PRINTGL 1.00 REFERENCE

(C) Copyright Cary Ravitz 1990

Cary Ravitz

P. O. Box 25068

Lexington, KY  40524-5068

g-1

CONTENTS

INTRODUCTION ..................................................... 2

LICENSE AND REGISTRATION                                           2

INSTALLATION ..................................................... 3

OPERATION                                                          3

SYNTAX ........................................................... 4

/A - LOCATION OPTION                                               5

/C - PEN COLOR OPTION ............................................ 5

/D - OUTPUT DESTINATION OPTION                                     5

/F - OUTPUT FORMAT OPTION ........................................ 6

/I - SCALING POINT LOCATION OPTION                                 6

/L - PAGE LAYOUT OPTION .......................................... 7

/M - MAGNIFICATION OPTION                                          7

/N - PAGE NUMBER OPTION .......................................... 8

/O - ORIGIN AND ORIENTATION OPTION                                 8

/P - PLOTFILE OPTION ............................................. 8

/S - PEN SHADING OPTION                                            8

/T - TEMPORARY FILE OPTION ....................................... 8

/W - PEN WIDTH OPTION                                              9

/Y - Y/D SWITCH SETTING OPTION ................................... 9

/Z - CHORD ANGLE OPTION                                            9

EPSON 9 PIN PRINTERS ............................................ 10

EPSON 24 PIN PRINTERS                                             10

HP LASERJET ..................................................... 10

HP PAINTJET                                                       10

IBM LASERPRINTER ................................................ 11

IBM PROPRINTER                                                    11

IBM PROPRINTER X24 COMPATIBLE PRINTERS, IBM QUICKWRITER ......... 11

IBM PROPRINTER X24                                                11

IBM QUIETWRITER 2 AND 3 ......................................... 11

CGA DISPLAY                                                       12

EGA DISPLAY ..................................................... 12

HERCULES DISPLAY                                                  12

MCGA DISPLAY .................................................... 12

VGA DISPLAY                                                       12

BIT MAPS ........................................................ 12

PLOTFILE SPECIFICS                                                13

LINE TYPE PATTERNS .............................................. 14

COMPATIBILITY AND TECHINCAL INFORMATION                           15

WHAT TO DO IF YOU HAVE PROBLEMS ................................. 15

g-2

INTRODUCTION

PrintGL prints an HP-GL (Hewlett-Packard Graphics Language) plotfile

on a matrix printer. HP-GL is HP's pen plotter control language that

is widely  supported by graphics programs including AutoCAD, GenCAD,

MathCAD, SAS, MICROCADAM, Schema, and many more.  PrintGL has native

mode drivers for Epson compatible 9  and 24 pin dot matrix printers,

HP   LaserJet,  DeskJet,  and  PaintJet,  and   IBM  9  and  24  pin

Proprinters, Quietwriter 2 and  3, and  LaserPrinter.  It will  also

display plots with  a CGA, EGA, VGA, enhanced VGA, or HGC and output

a bit map.

Even if  your graphics program  supports your printer,  you may find

that  PrintGL  gives  better print quality.  PrintGL  uses  the best

graphics  modes available for each printer that it supports.  If you

need  printed graphics output from personal software, you can output

HP-GL and use  PrintGL to  do the  printing.  This  gives  immediate

support to a wide range of printers.

PrintGL interprets all of the HP 7470 command set that is applicable

to plotfiles.  None  of  the  digitize,  output,  or  device control

commands (except plotter on and off) are supported as these are only

used with direct control of a plotter. And the error mask command is

not supported.  In addition to the HP 7470 commands, PrintGL handles

paper feed commands and eight pens (instead of two).

If  you are interested in  using PrintGL  as a printer  driver  in a

commercial program, contact me. I can prepare a special version with

a  program  interface, alternate  vector  language input,  alternate

font, or support for additional printers.

LICENSE AND REGISTRATION

You may use PrintGL without charge on a trial basis to determine its

suitability for  you.  If  you  continue to  use PrintGL  after your

evaluation a $25 registration is requested.

PrintGL is  provided as is.  There  are no warranties  expressed  or

implied.

You may distribute PrintGL if you keep  the entire package together,

unchanged,  including this  license explanation, and  do  not charge

more than a small handling fee ($10).

To  register your copy of PrintGL,  send your name, address, and $25

check to:

Cary Ravitz

P. O. Box 25068

Lexington, KY  40524-5068

You will  not  receive  a  new copy of  PrintGL.  The bulletin board

distribution package is the complete, full function program.

g-3

INSTALLATION

The PrintGL package consists of these files:

PRINTGL.EXE  program

PRINTGL.DOC  documentation

SAMPLE.PLT   sample plotfile

To  install PrintGL copy the distribution files to your working disk

or directory.

PRINTGL.EXE is a stand alone executable  file.  There  are no  extra

printer driver files or overlays.

PRINTGL.DOC is this manual.  You can print it with "COPY PRINTGL.DOC

PRN" (16 pages).

SAMPLE.PLT is a sample plotfile to help you try out PrintGL.

OPERATION

PrintGL reads a plotfile, does arc and character to line conversion,

stores  the data in a coordinate list,  and if needed, puts the list

on disk.  It then reorients the  data and determines the plot  size.

Next the data is resized and relocated.  Finally the coordinate list

is rasterized  and printed  in swaths.  You are likely  to  see  the

printer print  a swath and stop while PrintGL is processing the next

swath.

When you display a plot, press Esc to return to the DOS text screen.

Ctrl-Break  stops  PrintGL at the next break  point.  Ctrl-Break  is

ignored when PrintGL is waiting for user input.

If  your  plotfile  contains  any  IW  (input window)  commands then

PrintGL does any automatic centering and sizing (as specified by the

/A  and /F options) assuming that you want to see the entire window,

even if there is no data in the window.

PrintGL  works within a printer's current  margins and starts at the

current print position.  If you decline  the form  feed suboption of

the  /L option then PrintGL  leaves the printer at the  start of the

line directly below the plot.  All printing is done in printer lines

that  cover the full width of the printhead.  This is noted  in  the

screen output so  that you can determine exactly how much  space the

plot takes. For example, if you specify a 3 inch high plot (with the

/L option) with a 24 pin  printer,  PrintGL notes "23  lines  24/180

inch/line", so the actual height of the plot is 3.067 inches.

Internally PrintGL uses two byte coordinates on a 1016 dots per inch

grid. This allows for plots with coordinates of over 30 inches - far

larger than an HP7470 can handle.  However  if you are using PrintGL

to emulate a large plotter, you may overflow the internal coordinate

system.  This  condition  is not checked, and it will cause  garbage

output.  You can change the internal resolution to 508 dots per inch

(giving 60  inch maximum  plot sizes) by  editing PRINTGL.EXE with a

binary editor -  search for the  string "indpi"  and change the next

two bytes from hex F803 to hex FC01.

g-4

SYNTAX

The calling syntax of PrintGL is:

PRINTGL plotfile /option1 /option2 ...

The  plotfile  may  be  specified  with  the  /P  option  if  it  is

inconvenient to make it the first parameter.

If a given option is entered more  than once, only the last entry is

used.  And if  an  option  is invalid  (but  not  incorrect),  it is

ignored.  This  lets  you set  up a  batch  file with your preferred

options  but override them as desired.  For example,  create PHP.BAT

with the line:

PRINTGL /FL /M1 /P %1 %2 %3 %4 %5 %6 %7 %8 %9

Then  PHP can be called just  like PRINTGL  but  it  defaults to  HP

LaserJet format with magnification 1.

'-' may be used instead of '/' to prefix the options.  If you do not

enter  any parameters  or if  any  parameter  is  incorrect, PrintGL

displays its help screen.

Below [ and ] enclose optional suboptions  and �  separates mutually

exclusive suboptions. The options and suboptions may be in any order

except that multiple numeric suboptions are ordered. "c" refers to a

suboption  character,  "x"  and  "y"  to  suboption  floating  point

numbers, "i" and "j" to suboption integers, and "f" to a DOS file or

device name.  For integer input, floating point numbers are accepted

and  rounded  to  an  integer.  If an  option has  multiple  numeric

suboptions, these may be separated with commas or blanks.

********************************************************************

It  is very important to choose the right output format (/F option).

It is likely  that  the  default  output  format will  work on  your

printer, but it is equally likely  that it  will not produce optimal

results.  Please look carefully at the entire list of output formats

before  choosing one, and do not assume that because a  format works

that it is the best one to use.

********************************************************************

g-5

/A - LOCATION OPTION

/A[O][x,y] - location - default /A

The A option determines  the  location of the plot  in  the  printer

window, by specifying a point on the plot that is to be aligned with

a point in the printer window. The point on the plot is normally its

center,  but  you can specify  a point with x,y  in  inches from the

plot's  origin.  The  point  in  the printer window is  normally the

center, but O says to make it the origin.  The  default /A  puts the

center of the plot at the center of the printer window.

When PrintGL is  finding the center of a plot, the  entire rectangle

defined  by any IW command  is included  in the overall  size of the

plot.

/C - PEN COLOR OPTION

/Cc.. - color - default /CL

The C  option specifies  the color of each  of the eight  pens.  The

colors are  B  for blue, C for cyan, G for green, L for black, M for

magenta, R for red, and Y for yellow.  Unspecified pens use the last

specified color  (/CRGB is the  same as /CRGBBBBBB).  If  the output

format does not allow color then this option is ignored.

The following output formats handle colors:

/FB  bit map

/FE  EGA

/FN  Epson 9 pin

/FP  HP PaintJet

/FT  Epson 24 pin

/FV  VGA

/FW  alternate VGA

Color  may  be  emulated on  black only  printers  with shading  (/S

option).

/D - OUTPUT DESTINATION OPTION

/Df - destination - default /D1

The D option specifies the output printer port, device, or file.  1,

2, or  3 causes PrintGL  to use the BIOS printer routines, bypassing

DOS.  This is  normally faster than specifying the DOS device (LPT1,

LPT2, LPT3), but  it  may  cause  problems with  print  spoolers  or

multitasking systems.

Display output  can only go to the screen and causes this  option to

be ignored.

g-6

/F - OUTPUT FORMAT OPTION

/Fc[+�-�*�i,j] - output format - default /FG

The F option specifies which output  format to use.  The  format  is

chosen with a device character and  a resolution modifier (-, +, *).

*, +, and - are allowed even where they have no affect. For bit maps

you can choose a specific  resolution in dots per inch.  Resolutions

here are given in horizontal x vertical dots/inch.

To return to DOS from a displayed plot, press Esc.

/F1     IBM Proprinter ..................................... 120x72

/F1+    IBM Proprinter (2 pass)                              120x144

/F2     IBM Quietwriter 2 .................................. 240x240

/F3     IBM Quietwriter 3                                    240x240

/F4-    IBM LaserPrinter ................................... 150x150

/F4     IBM LaserPrinter                                     300x300

/F5     IBM Proprinter X24 compatible printers ............. 180x180

/F5+    IBM Proprinter X24 compatible printers (2 pass)      360x180

/F6     IBM Proprinter X24 ................................. 180x182

/F6+    IBM Proprinter X24 (2 pass)                          360x182

/FB     bit map ............................................ 100x100

/FBi,j  bit map                                                ixj

/FC     CGA display ......................................... 62x25

/FE     128K EGA color display                                62x45

/FH     Hercules graphics card display ...................... 70x45

/FL-    HP LaserJet compatible printers                      150x150

/FL     HP LaserJet compatible printers .................... 300x300

/FM     128K EGA monochrome display                           62x45

/FN     Epson 9 pin compatible printers .................... 120x72

/FN+    Epson 9 pin compatible printers (3 pass)             120x216

/FN*    Epson 9 pin compatible printers (6 pass) ........... 240x216

/FP     HP PaintJet                                          180x180

/FT     Epson 24 pin compatible printers ................... 180x180

/FT+    Epson 24 pin compatible printers (2 pass)            360x180

/FT*    Epson 24 pin compatible printers (4 pass) .......... 360x360

/FV     VGA display                                           62x62

/FV+    enhanced VGA display 720x540 mode ................... 70x70

/FV*    enhanced VGA display 800x600 mode                     78x78

/FW     VGA display ......................................... 62x62

/FW+    alternate enhanced VGA display 800x600 mode           78x78

/I - SCALING POINT LOCATION OPTION

/IA�R�W[N] - IP point location - default /IA

The I  option sets the  initial scaling points (P1, P2 - the scaling

points may  be  set  in  the HP-GL  file  with the  IP  command).  A

(absolute) sets the points to 250,279 and 10250,7479 (the same as an

HP 7470). R fits P1 and P2 to the plot window with at least .25 inch

margins and an x/y ratio of 10/7.2 (the same ratio as the  HP 7470).

W fits P1 and P2 to the plot window with a .25 inch margin.  N makes

the margin 0 for R or W and is ignored with A.

g-7

/L - PAGE LAYOUT OPTION

/L[B][F][R][x,y[,x,y]] - page layout - default depends on printer

The L option defines the page layout.

B boxes the plot at the hard clip limits using pen 8. This generally

increases the amount of data  sent to a printer, but not bit maps or

displays.

F  causes a form feed after the plot.  This  is ignored for bit maps

and display output.

R causes reverse  video (black on white)  on one color display plots

and bit maps and is otherwise ignored.

The first optional x and  y are the  plot width and height in inches

rounded to .01.  These define the hard clip limits. These values are

not checked against your printer's capabilities.  If  you specify  a

page  layout larger than the  output  device can handle, the results

are unknown.  You may need to reset your  printer's margins, maximum

line length, or page length to get large plots.  The size needed for

a given width and height exceeds x and y by 8 pels to allow for line

widths.

The  second optional x and y are the left and top margins,  measured

from the printer's left and top margins to the hard clip limits. The

left margin  is rounded to .01 inch.  The top margin is rounded to a

1/6  inch  and  line  feeds  (assumed to  be 1/6 inch)  are  used to

position the paper.  The margins  are subject to printer  resolution

and  efficiency considerations,  and may not be  exact.  Margins are

ignored for display output and bit maps.

Each printer has a default page layout. The default width and height

are  7.53x10.14  for  printers  and  bit  maps  and  10.14x7.53  for

displays.  The  margins  are  printer  specific  to  compensate  for

different default  margins.  In  evaluating  user  options,  PrintGL

processes each option and when finished reprocesses the default page

layout  for  the  last chosen printer and then  reprocesses the last

user /L option.  This  procedure gives good default  values so  that

your /L option can change the  letter or  size  suboptions and still

have good margins.

/M - MAGNIFICATION OPTION

/M[F]x - magnification - default /MF.9

The M option specifies the magnification.  F  fits the  plot to  the

printer window size.  The default is  /MF.9 which makes the plot 90%

of the largest size that will fit in the printer window.

g-8

/N - PAGE NUMBER OPTION

/N[i] - page number - default /N

Ni says to print only page i of the plotfile.  If you do not specify

i then all pages in the plot will be printed (this is the default).

/O - ORIGIN AND ORIENTATION OPTION

/O1�2�3�4[L] - origin, orientation - default /O1 or /O2

The O options sets the printer's origin and orientation.  The origin

is  one of four corners numbered  1..4 for upper  left, lower  left,

lower  right,  and upper  right.  The  L suboption  specifies a left

handed coordinate  system (this should never be  needed).  1  is the

default  origin for  printers and  matches the  plotter's coordinate

system.  2 is the default origin for displays which also matches the

plotter's coordinates due to the reversed width/height ratio.

/P - PLOTFILE OPTION

/Pf - plotfile - no default

The  plotfile  may  be specified  as the first parameter without any

prefix, or  it may be specified anywhere in the option list with the

/P prefix.

/S - PEN SHADING OPTION

/Sc.. - pen shading patterns - default /S0

This option assigns a shading pattern to each pen.  Unspecified pens

use the last specified shade.  The  shade values are 0..7.  0 is the

default  solid line.  1..4 are 1 dot patterns (75%, 50%, 25%, 12.5%)

and  need  line widths of at least three to give good looking lines.

5..7 are 2x2 dot patterns (75%, 50%, 25%) and need line widths of at

least five  to give good looking lines.  Some printers may wash  out

shading because they  have a dot size  that is large relative to the

dot spacing.  This is especially  true with dot  matrix printers  in

multipass modes.

1         2         3        4         5         6         7

��������  ��������  � � � �  �   �     ��������  ��  ��    ��  ��

��������  ��������  � � � �    �   �   ��  ��      ��  ��

��������  ��������  � � � �  �   �     ��������  ��  ��    ��  ��

��������  ��������  � � � �    �   �   ��  ��      ��  ��

/T - TEMPORARY FILE OPTION

/Tf - temporary file - default /TPRINTGL.TMP

PrintGL uses a temporary file when it does not have enough memory to

handle a  plotfile.  It does  not use extended or expanded memory so

you can best utilize this memory by  putting the temporary file on a

RAM disk  in  the  extra  memory.  If you have 640K and average size

plotfiles then  it is likely that PrintGL will never use a temporary

file.

g-9

/W - PEN WIDTH OPTION

/Wc.. - pen widths - default /WA

The W option assigns a line width to each pen (1..8). Each width may

be  1..8 pels or A..H for the default  width of the  selected output

format plus 0..7 (maximum 8).  Unassigned pens use the last assigned

width.

The default line  width  is 1 for all output formats except  240 and

300 dpi printers which have a default line width of 2.

/Y - Y/D SWITCH SETTING OPTION

/YD�Y - Y/D switch setting - default /YD

The Y option sets the plotter's rear panel Y/D switch to D  or Y.  Y

starts with the plotter off and responds to esc.(, esc.Y, esc.), and

esc.Z  commands.  D  starts with  the plotter  on and  ignores these

commands.

/Z - CHORD ANGLE OPTION

/Zi,j - default and minimum chord angle - default /Z15,5

The HP 7470 defaults  to a  chord  angle of  5  and has  no minimum.

Because  PrintGL stores each  chord  in a  circle  in  its  internal

database, this can be  very  time consuming.  And because of the low

resolution   of  printers  it  does   not  produce  a  corresponding

improvement in print quality. The Z option specifies the default and

minimum  chord angles.  The default is /Z15,5.  For better emulation

use /Z5,0.

g-10

EPSON 9 PIN PRINTERS

/FN covers a wide range  of  9  pin dot matrix printers.  You need a

cyan/magenta/yellow ribbon  to  get  color  prints.  /FN+ uses 1/216

indexing and /FN* uses 1/216 indexing and 1/240 graphics.

The IBM  9  pin  Proprinters do not do  1/216  indexing and  do  not

correctly handle 1/240  graphics commands.  /FP  is a special driver

for Proprinters.

EPSON 24 PIN PRINTERS

/FT covers the entire line of Epson LQ printers and  many compatible

printers  (NEC,  Panasonic,  etc.).  You need  a cyan/magenta/yellow

ribbon to get color prints. /FT+ uses 1/360 graphics mode. /FT* uses

1/360 graphics mode and 1/360 indexing and resets the printer's line

feed distance to 1/6 inch on completion.

These  drivers will not  work with IBM  Proprinter X24s in alternate

graphics mode because they use the absolute tab command.

HP LASERJET

/FL works with any LaserJet or DeskJet.

This driver assumes that your printer is in portrait mode.

At  300 dpi (the default), if you have just 512K  in a LaserJet then

your  plot can  cover about  half  the area of  the  page.  1.5M  is

sufficient for  a  full page of graphics.  At  150  dpi (/FL-)  512K

handles  a full  page.  (All  DeskJets  can  handle a full  page  of

graphics at 300 dpi.)

/FL- defaults to a page  height of 10.11 inch instead of 10.14 inch,

so that plots fit on a page without changing margins.

HP PAINTJET

/FP should work with  any 180  dpi HP-PCL  printer, including the HP

Rugged Writer, but this has not been tested.

g-11

IBM LASERPRINTER

/F6 drives the  LaserPrinter  in  native mode.  /FL will work if the

LaserPrinter is in HP LaserJet mode.

At 300 dpi (the default), if you have just 512K in your LaserPrinter

then your plot can cover about half of the page.  1.5M is sufficient

for a full page of graphics.  At 150 dpi (/F6-) 512K  handles a full

page.

IBM PROPRINTER

The /FP driver  is  for all 9 pin  Proprinters.  The high resolution

driver (/FP+) uses  1/144  inch indexing  and it does  not work with

Epson 9 pin compatible printers that have 1/216 inch indexing.

There  is no  /FP*  resolution,  which  would  be  240x144,  because

Proprinters do not handle 1/240 graphics correctly.

IBM PROPRINTER X24

The /F5 driver covers all Proprinter  X24s.  These printers  can use

the X24 compatible driver (/F4), but because they do 1/144 indexing,

the print will have horizontal white streaks every 2/3 inch. The /F5

drivers  compensate  for  the  indexing  by  defining  the  vertical

resolution as 182 dots per inch  and  indexing 19/144 inch per line.

To work properly the printer  must be  on a  1/144 boundary when the

graphics are printed.  If you keep the line feed distance a multiple

of 1/144 (1/6, 1/8, 1/9) then this will always be the case.

Proprinter X24 and  XL24  are very  slow with  graphics, making many

passes per line.  This limitation cannot be  overcome  with software

(to the best of my knowledge).  The X24E and XL24E are significantly

faster, and are much preferred to the original X24s.

IBM PROPRINTER X24 COMPATIBLE PRINTERS, IBM QUICKWRITER

The IBM Quickwriter is  a  Proprinter X24 compatible printer  except

that  it does 1/180 indexing.  This is  probably the  case with most

Proprinter X24 compatible printers. /F4 is the correct driver to use

for  such  printers.   /F5  (the  Proprinter  X24   driver)  is  not

recommended.

IBM QUIETWRITER 2 AND 3

The Quietwriter  drivers reset the  line  feed distance to 1/6  inch

upon completion.

g-12

CGA DISPLAY

The  CGA  driver (/FC)  does not give  color.  The  CGA card has  no

graphics mode that supports the 7 colors used by PrintGL.

EGA DISPLAY

The EGA drivers (/FE, /FM) require a 128K or 256K EGA. If you have a

64K EGA, the CGA (/FC) driver will work.

/FM is for an EGA card with  a digital monochrome  monitor attached.

If  your display  adaptor supports  Hercules  modes, /FH  will  give

better resolution.

HERCULES DISPLAY

/FH supports the  Hercules  monochrome graphics card.  There  is  no

support for the Hercules InColor card.

MCGA DISPLAY

/FV will work with an MCGA if you do not use color.

VGA DISPLAY

The  /FV+  output  format requires a VGA card that  supports 720x540

mode.  The  /FV* output  format  requires a  VGA card  and multisync

monitor  that  support  800x600 mode.  You may need to run a special

driver that is supplied  with  the card to get these modes.  PrintGL

sets up the screen by  setting BIOS display modes hex 63 and hex 64.

Your VGA card must use these modes to work with /FV+ and /FV*.

/FW+ is the  same as /FV* except that it uses BIOS display  mode hex

58 to set 800x600 mode.  This works with Paradise enhanced VGA cards

with multisync monitors. /FW is identical to /FV.

BIT MAPS

Color bit maps are two pels per byte.  Each  pel nibble  = 1*blue  +

2*green + 4*red +  8*intensity.  The  intensity is 0 for black and 1

otherwise.  The  background is  15  - white,  which  is  not  a user

selectable color.

/CL  (default) bit  maps are 8  pels/byte with 1 meaning black and 0

background.  This may  be  reversed  with  the R suboption of the /L

option.

The  map size is the size specified  by the L  option plus 8 pels in

each direction to give room for line widths of up to 8 pels.

g-13

PLOTFILE SPECIFICS

In reading a plotfile, PrintGL ignores carriage returns, line feeds,

and nulls  except where  they may be valid characters (LB,  SM,  DT,

esc.), and ASCII EOF (dec 26) is interpreted as an end of file mark.

PrintGL's HP-GL specification is taken  from the HP 7470 programming

manual.  PrintGL supports  this  HP-GL  subset  except for  digitize

commands, output commands,  the error mask command, and most  device

control commands  (plotter  on  and  off  are supported).  Supported

commands  are listed  here.  Commands  that  are  not supported  are

ignored.

Two extensions are  also supported.  Form feed  commands are allowed

and 8 pens are allowed instead of 2.  To take advantage of the this,

it may be useful to use a 7475 (6 pens) or 7550 (eight pens and form

feeds) driver to create your plotfile.

AA x,y,aa[,ca]                     arc absolute

AF                                 new page (extension)

AR x,y,aa[,ca]                     arc relative

CA [0/1/2/3/4]                     designate alternate character set

CI r[,ca]                          circle

CP [sp,ln]                         character plot

CS [0/1/2/3/4]                     designate standard character set

DF                                 set default values

DI [run,rise]                      absolute direction

DR [run,rise]                      relative direction

DTt                                define label terminator

IN                                 initialize

IP [p1x,p1y[,p2x,p2y]]             input p1 and p2

IW xll,yll,xur,yur                 input window

LB...text...t                      label

LT [pn,pl]                         designate line type and length

PA [x,y] [,x,y] ...                plot absolute

PD [x,y] [,x,y] ...                pen down

PG [i]                             new page (extension)

PR [x,y] [,x,y] ...                plot relative

PU [x,y] [,x,y] ...                pen up

SA                                 select alternate character set

SC xmin,xmax,ymin,ymax             scale

SI [width,height]                  absolute character size

SL [tan a]                         character slant

SM[c]                              symbol mode

SP [0/1/2/3/4/5/6/7/8]             select pen (2..8 extension)

SR [width,height]                  relative character size

SS                                 select standard character set

TL [[tp],tn]                       tick length

UC [[pc,]dx,dy] [[,pc],dx,dy] ...  user defined character

XT                                 x-axis tick

YT                                 y-axis tick

esc.(                              plotter on (only with /YY)

esc.Y                              plotter on (only with /YY)

esc.)                              plotter off (only with /YY)

esc.Z                              plotter off (only with /YY)

g-14

LINE TYPE PATTERNS

Below are PrintGL's line type patterns.  They  are close to, but not

identical to HP's.  Line type patterns are dynamically limited  to a

minimum length of 20 pels.

0        at endpoints only

1        �         �         �         �

2        �����     �����     �����     �����

3        ��������  ��������  ��������  ��������

4        ������� � ������� � ������� � ������� �

5        ������ �� ������ �� ������ �� ������ ��

6        ����� � � ����� � � ����� � � ����� � �

default  ����������������������������������������

Note  that line type  is  specified in an HP-GL plotfile with the LT

command - it is not a property of the pen that is being used to draw

the  line.  You  cannot specify  that  a  certain pen  should  use a

specific line type.  (Color, shading, and  width  are pen properties

and can be assigned to pens.)

g-15

COMPATIBILITY AND TECHNICAL INFORMATION

PrintGL will run on any IBM compatible MS-DOS computer. It will work

in  a  region  as small as 80K but  may need  more memory  for  high

resolution  or wide prints.  104K should suffice for the worst  case

black/white prints  -  13.2 inches wide, 360x360 dpi.  Color  output

requires  an additional  32K  or  more  for high  resolution /  wide

prints. 184K is needed for the worst case.  A larger region may make

things go  faster by allowing larger buffers and averting the use of

a spill file.

By default PrintGL uses the BIOS printer interrupt to send data to a

printer.  If  a  file or device is specified for output then DOS 2.0

file handling is  used instead.  PrintGL uses  DOS 2.0 file handling

for all  disk input and output.  Screen messages are done  with DOS,

graphics screen modes  (except Hercules) are set up  with  BIOS, and

graphics  screen updates are done by writing directly to the display

buffer. Keyboard handling is done via BIOS.

PrintGL takes the BIOS break interrupt (hex 1B) and the DOS critical

error interrupt (hex 24) in addition to those  taken  by  the  Turbo

Pascal 5.5 startup code.

PrintGL was compiled with Borland's Turbo Pascal 5.5.

WHAT TO DO IF YOU HAVE PROBLEMS

If  you  have problems with PrintGL or questions about it, there are

two  ways  to get  in  touch with me.  First is  my support bulletin

board.  Prepare a file  with the details  of your problem.  Call the

bulletin board at 606-268-0577  (1200/2400,N,8,1  24 hours/day)  and

upload  your file.  It will be assigned a question number - remember

this number.  I will  prepare  an  answer file, hopefully  within 48

hours. Call the bulletin board and download the corresponding answer

file to get your answer. All question and answer files are public.

You  can  also mail questions  on disk  to  me (360K, 720K, or 1.44M

diskettes  only - I  cannot  read  1.2M  diskettes).  Please include

return postage. Send the diskette to:

Cary Ravitz

P. O. Box 25068

Lexington, KY  40524-5068

I must have specific details. Do not assume that I know what you are

trying to do.